# Synapptic – Quick start guide (HM100)

Thank you for purchasing from RNIB. In the event the item is unsuitable, please contact us within 14 days of receipt to obtain your returns number. To ensure your return goes smoothly, the item and all components must be in as new condition and packed in its original, undamaged packaging. For further details and guidance on returning faulty items, please see the Terms and Conditions of sale and How to contact RNIB sections of this instruction manual.

Please retain these instructions for future reference. These instructions are also available in other formats.

## General description

A 6 inch mobile phone with pre-installed Synapptic Lite software. Synapptic software allows for ease of use, and audible guidance throughout the phone. This is a quick start guide but there is also a blue question mark in the top right of each screen if you get stuck, for audible guidance. You can locate this by sliding your finger across the screen to the top right corner.

Each purchase of Synapptic entitles you to one hour of free training and unlimited UK technical support. Please contact Synapptic on 0191 909 7 909 for to arrange a suitable date and time for the training, or for free technical support.

### Items supplied with Synapptic phone

Phone.

Pre-installed software.

Charging wire.

SIM card with £10 credit.

Memory stick adaptor.

## Using the product

### Inserting a SIM Card

Your phone comes pre-configured with a free SIM card, mobile telephone number and £10 credit. (Please see the separate sheet sent with the phone for further details).

If you already have a SIM card and mobile phone number from a previous phone, you can replace the pre-configured SIM card with your own.

To insert your SIM card into the phone, some sighted assistance may be required. Ensure the phone is powered off then remove the SIM card tray on the left-hand edge of the phone. To remove the tray, insert a pin or the end of a paperclip into the small hole located approximately 1 inch below the top left edge of the phone.

Slide the SIM card into the appropriate slot, ensuring it's the correct way around.

After inserting the SIM card into the tray, replace the tray back into the original slot and push into place.

### SIM Card sizes

Three sizes of SIM card are available (regular, micro and nano). If your SIM card is too big, contact your network provider, who will be able to issue a smaller version, usually for free. The Synapptic phones accept a nano sized SIM card.

Note: SIM card Pin security numbers are not supported.

### Charging the device

To charge the phone, insert one end of the provided cable into the USB / charging socket of the phone. For the phone this is located on the bottom edge of the device, in the middle. The other end of the cable is plugged into a suitable USB-C mains power adaptor, or the USB-C socket of any computer.

Please note: Do not force the charging cable into position as this could damage the socket.

It is recommended to charge the phone each night. We also recommend charging the phone fully before using it for the first time.

### Turning the device on and off

The phone has three power states: fully on, fully off and standby. When in standby mode, the screen will appear off, but the device will still be receiving power, ready to receive calls or emails, etc.

In standby mode the device will use very little power and can last a week or more on a single charge. Standby mode will be activated if the device is not used for a number of minutes.

### Turning the device on

To turn the device on, when it is fully off, press and hold the power button on the edge of the device for three seconds. (For the phone this is the smaller of the tactile buttons on the right-hand edge of the device. After a moment, a short sound will play and it will launch the main menu about 30 seconds later.

If the device is in standby mode, just press the power button very briefly. The device will turn on instantly.

When using the phone, the key-guard screen will display when powering on. This screen helps to stop your pocket or bag from operating the phone if the power button is pressed accidentally.

To unlock the key-guard screen, swipe your finger from left to right across the whole width of the screen (anywhere up or down the screen). When successfully unlocked a bell will sound.

### Turning the device off

Press the power button on the edge of the device briefly to turn off the screen and place the device in standby mode.

It's normal practice to leave the device on standby and not to switch it off fully.

To fully turn off your device select the Turn Off option on the Quit Synapptic menu, located at the end of the Main Menu. The Android Power Off screen will display and Synapptic Reader will speak out the various menu options. These are usually Power Off, Restart and Emergency mode.

Select the Power Off option, then select it again to confirm.

### Operating the menus and using the touch screen

All Synapptic screens have a consistent look, feel and operation. Once you have learnt how to use one menu, all other menus can be used just as easily.

The following describes how to operate the menus with their default factory-supplied settings. These include voice output, yellow text on a black background and a magnification level giving four menu items on each screen.

The structure of the main menu is very simple. At the top is the Menu Title, under which is a Previous Page button, a number of menu options, then finally a Next Page button at the bottom of the screen.

To operate the menu, place your finger at the very top of the screen and keep it gently pressed down on the screen.

Move your finger down the screen and do not lift up your finger until you have found the menu option you wish to select. Lifting your finger off the screen will indicate to the system that you wish to select this option. If you do not wish to select any option, just slide your finger back off the top of the screen.

As your finger moves onto each menu option, a beep will sound and the option will be both highlighted and spoken out.

After selecting an option from the main menu, subsequent screens and menus will have a slightly different layout.

Each screen and menu, following from the main menu, has an additional red cross in the top left corner and an additional blue and white question mark in the top right corner.

Use the red cross in the top left to cancel the current screen or menu and go back to the previous screen or menu.

Use the blue and white question mark in the top right corner of a screen to show help specifically for that screen.

### Answering and ending phone calls

When the phone rings, use the green Answer button in the bottom left of the screen to answer the call. Use the red Reject button in the bottom right to reject the call. Alternatively, press the volume-up button to answer a call and the power button to end a call. The volume-up button is top of the larger tactile button on the right edge of the phone. The power button is the smaller tactile button.

### Customisation

Use the Settings menu to customise the following features:

colour scheme (text and background colour)

icon colour and contrast

screen brightness

zoom level and text size

main menu items (add, delete and re-order)

main menu as a single page grid or multi-page list

voice volume

voice speed

ringtones and notification tones

key sounds, beeps, vibrations and other alerts

lock screen.

Top tip! We recommend experimenting with different zoom levels, as menu layouts change slightly between high and low zoom. This allows some text to rearrange and become larger than normal.

### Wi-Fi and mobile Internet connections

The Synapptic Lite Software installed on this phone has a reduced set of features. Many of the Internet related features found in the full version of the software are removed for simplicity. However, an Internet connection is still recommended so that software updates can be downloaded when needed.

We recommend using a Wi-Fi connection whenever possible. This could be your home Wi-Fi connection or a Wi-Fi connection in a cafe, shop or hotel.

To setup Wi-Fi, select the Settings option on the main menu. Then, on the Settings menu select the Wi-Fi option. The Wi-Fi menu has options for turning Wi-Fi on and off and selecting a particular Wi-Fi access point.

If a password is required for the selected Wi-Fi connection, Synapptic will prompt for it. This password will be remembered and the connection automatically made when next in range.

To remove a password from a particular Wi-Fi connection, select the connection in the Wi-Fi list, then select the Forget option.

### Mobile Internet connection (via SIM card)

When not connected to a Wi-Fi Internet connection, smartphones may automatically connect to chargeable mobile phone internet connections to keep themselves updated and synchronise contacts or check for new emails, etc.

Please check with your SIM card provider (Orange, Vodafone, O2, 3, etc.) about data charges that may be incurred when using a mobile phone's 3G / 4G or other internet data connection. We also recommend you ask the SIM card provider to turn off roaming internet when travelling abroad, as this may incur significant extra charges.

To avoid any problems with mobile internet data charges we recommend signing up to a contract or monthly Pay-as-You-Go deal, that includes an internet data allowance.

### Account settings

Your smartphone will arrive fully configured and ready to use. An enclosed settings sheet will detail the initial Google Account, Email address and Remote Support security number that the device has been configured with.

Any or all of these initial settings can be replaced with your own settings if required.

## Software updates

Synapptic software comes with one year of free updates, giving you access to new and updated features, along with security and compatibility updates. Optionally, when this expires, further years of updates can be purchased. (One purchase covers all your Synapptic devices).

To update Synapptic, select the Phone Settings option from the main menu, then select the Updates option.

We recommend updating Synapptic at least once every six months.

## Synapptic help and support

There are five ways to get help and support with Synapptic

Help option on the main menu. The help option on the main menu contains basic information about the operation of the Synapptic system. A search facility is also available.

Screen help. Each screen has a blue and white question mark in the top right corner. Select this to view help documents and video tutorials for the current screen.

Synapptic website. The Synapptic website has additional help, news and support information. Visit www.synapptic.com for details.

Lifetime free UK telephone support – Call us for help with setting up, using, or updating your Synapptic product.

Free training. Each purchase of Synapptic entitles you to one hour of free training. Please contact Synapptic on 0191 909 7 909 to arrange a suitable date and time for the training.

## How to contact RNIB

Phone: 0303 123 9999

Email: shop@rnib.org.uk

Address: RNIB, Midgate House, Midgate, Peterborough PE1 1TN

Online Shop: shop.rnib.org.uk

### RNIB Technology Team

Phone: 0207 391 2280

Email: tfl@rnib.org.uk

## Terms and conditions of sale

This product range is guaranteed from manufacturing faults for 24 months from the date of purchase.  If you have any issues with the product and you did not purchase directly from RNIB then please contact your retailer in the first instance.

Please contact RNIB for information on how to return in the unlikely event that your product becomes faulty. This will help us to deal efficiently with your product return.

You can request full terms and conditions from RNIB or view them online.

RNIB Enterprises Limited (with registered number 0887094) is a wholly owned trading subsidiary of the Royal National Institute of Blind People ("RNIB"), a charity registered in England and Wales (226227), Scotland (SC039316) and Isle of Man (1109). RNIB Enterprises Limited covenants all of its taxable profits to RNIB.

This product is CE marked and fully complies with all applicable EU legislation.

Please do not throw items marked with this symbol in your bin.  Recycle your electricals and electronic devices free at your local recycling centre. Search for your nearest recycling centre by visiting www.recyclenow.com.

### Why recycle?

Unwanted electrical equipment is the UK’s fastest growing type of waste.

Many electrical items can be repaired or recycled, saving natural resources and the environment.  If you do not recycle, electrical equipment will end up in landfill where hazardous substances will leak out and cause soil and water contamination – harming wildlife and human health.

RNIB are proud to support your local authority in providing local recycling facilities for electrical equipment.

To remind you that old electrical equipment can be recycled, it is now marked with the crossed-out wheeled bin symbol. Please do not throw any electrical equipment (including those marked with this symbol) in your bin.

### What is WEEE?

The Waste Electrical or Electronic Equipment (WEEE) Directive requires countries to maximise separate collection and environmentally friendly processing of these items.

### How are we helping?

In the UK, distributors including retailers must provide a system which allows all customers buying new electrical equipment the opportunity to recycle their old items free of charge. As a responsible retailer, we have met the requirements placed on us by financially supporting the national network of WEEE recycling centres established by local authorities. This is achieved through membership of the national Distributor Take-back scheme (DTS).

Date: March 2025.

© 2025 Royal National Institute of Blind People.