# HandyReader HD (HC122)

Please retain these instructions for future reference. These instructions are also available in other formats.

Contents

General description	2

Items supplied with the HandyReader HD	2

Orientation	2

Front	2

Back	3

Top edge	3

Bottom edge	3

Getting started	3

Using the product	4

Rechargeable battery	4

Inserting the battery	4

Charging the battery	4

Low-battery warning	4

Turning on/off	5

Auto power off	5

Reading	5

Adjusting magnification	5

Changing colour mode	5

Preferred colour mode	5

Freeze frame	6

Adjusting screen brightness	6

Turning LED lights on & off	6

Saving of settings	6

Flip-out stand	6

TV connection	7

Important precautions	7

Technical specification	8

How to contact RNIB	9

Terms and conditions of sale	9

## General description

The HandyReader HD is a handheld video magnifier that has tactile controls to make it simpler to operate. The high contrast, 3.5 inch screen displays up to 24x magnification. It also features a freeze frame function, so you can capture an image and then use the zoom function to enhance the text as needed.

## Items supplied with the HandyReader HD

- HandyReader HD Portable Digital Video Magnifier

- Rechargeable Battery

- Mains Power Adapter & USB Charging Cable

- Wrist Lanyard

- Storage Pouch

- Screen Cleaning Cloth

- Video Output Cable

- User Manual

- Quick Reference Guide

## Orientation

Hold the unit in front of you so that the LCD screen is facing towards you. If the HandyReader HD is in the correct position, there will be two buttons along the top edge.

### Front

On the front, and positioned towards the left hand side, is the LCD display.

Just below this are two LEDs. The LED on the left is the green power indicator. The one on the right is the red charging indicator.

To the right of the screen, along the top, are two buttons with a plus and a minus sign. These are the zoom out (minus, on the left) and zoom in (plus, on the right) buttons.

Below this is a cross symbol; this is the reading target, with the reading camera being situated below this cross on the back of the HandyReader HD.

Underneath the cross is a button with the letter ‘M’. This is the colour mode button. Tap to cycle viewing modes (colour combinations), press & hold to limit viewing modes to the currently selected colour combination & its inverse.

### Back

Turn the unit over so that the screen is facing down, with the power and freeze buttons still on the top edge.

On the left hand side is the camera lens, and just below this are a series of LED lights.

To the right of the camera lens is a flip-out stand which can be used to lower the minimum magnification level & assist with handwriting. Underneath this is the speaker.

On the right hand side of the back of the unit is the battery compartment cover. The serial number of the unit is located inside of this, should you need it.

Inside the battery compartment is the rechargeable Lithium-Ion Battery.

### Top edge

Along the top edge are two buttons. The button to the left is the power button. Press & hold (2 seconds) to turn On/Off. The button to the right is the freeze button. Tap to freeze the image on the screen, press & hold (2 secs) to turn sound On/Off.

### Bottom edge

Along the bottom edge are two sockets. The circular socket in the middle of the bottom edge is the video output socket (use only supplied cable). The socket to the right of this is the USB charging socket (use only supplied charger).

On the right hand corner is the wrist lanyard attachment point.

## Getting started

When new, both the high contrast screen and the camera have protective films over them. To improve image quality, these can be removed by peeling off the covers from one corner.

These may have been removed entirely by your distributor before shipping or have the corners folded down to make them easier to pull off.

## Using the product

### Rechargeable battery

### Inserting the battery

From the factory, the li-ion battery is not fitted in the unit. Shipping laws in your country may prevent your local distributor from inserting the battery for you prior to shipping. If you need to insert it, follow these steps:

Remove the Battery Cover by sliding outwards.

Insert the battery ensuring that the gold terminals in the battery compartment align with the terminals on the battery. Refit the battery cover.

### Charging the battery

Before using your HandyReader HD for the first time, it is important to fully charge its battery:

Connect the supplied mains charging adapter to the HandyReader HD using the supplied USB cable.

Connect the charger to a mains socket and turn on the power.

The LED Charging Indicator illuminates red to show charging is taking place.

When charging is complete the LED Charging Indicator will turn off.

Depending on your screen brightness setting, the HandyReader HD will now run for approximately 3.5 hours before displaying a low-battery warning.

When the low-battery warning is displayed, please follow the steps above to recharge the battery. It is possible to recharge the battery before the low-battery warning is displayed.

### Low-battery warning

When the HandyReader HD’s battery requires a recharge, a low-battery icon will be displayed on the High Contrast Display Screen. The green LED Power Indicator will also begin to flash when the low-battery warning appears.

## Turning on/off

To turn the HandyReader HD on, press & hold the Power Button for 2 seconds. The Green LED Power Indicator and High Contrast Screen will illuminate. If the sound is turned on, an audible tone will also be heard. To turn the unit off, simply press the Power Button again for 2 seconds.

### Auto power off

If the unit is not used for 2 mins, the unit will automatically power itself off to conserve battery.

## Reading

Place the HandyReader HD onto the information you wish to view. The camera is located to the right of the High Contrast Screen under the Reading Target to locate the correct position. Simply slide the HandyReader HD over the document / object, across the text to read.

## Adjusting magnification

Use the Zoom In & Zoom Out Buttons to adjust the magnification level. The HandyReader HD features continuous zoom from 2x to 24x.

## Changing colour mode

Tap the Colour Mode Button to cycle and select the desired viewing mode. The HandyReader HD has 9 viewing modes including 8 enhanced modes that drastically improve contrast when reading text. The modes are:

- Full colour – Ideal for photographs & diagrams in their natural colours

- Enhanced Black on White

- Enhanced White on Black

- Enhanced Black on Yellow

- Enhanced Yellow on Black

- Enhanced Black on Green

- Enhanced Green on Black

- Enhanced Blue on Yellow

- Enhanced Yellow on Blue

### Preferred colour mode

To store your preferred enhanced 2 colour mode, first cycle to your preferred 2 colour mode & then press & hold the Colour Mode Button for 3 seconds. The Colour Mode icon will be displayed on screen. Now when you tap the Colour Mode Button you will only cycle through your chosen colour mode, its inverse & full Colour.

To re-enable all Colour Modes, press and hold the M Colour Mode button for 3 seconds again.

## Freeze frame

To freeze the image on the screen, tap the Freeze Button. This is ideal if you are reaching up to read the price of an item on a shelf for example.

To un-freeze the image, simply tap the Freeze Button again.

## Adjusting screen brightness

Press & Hold the Colour Mode Button & tap the Zoom In or Zoom Out buttons to adjust screen brightness.

There are 5 levels of screen brightness: 20%, 40%, 60%, 80% & 100% (default). Using a lower screen brightness increases battery life.

## Turning LED lights on & off

The HandyReader HD features integrated LED lighting (on by default). When looking at shiny surfaces from a short distance (e.g. mobile phone screens) you may be able to improve image quality by turning the LEDs off. To do so, press & hold the Colour

Mode Button & Tap the Freeze Button

## Saving of settings

The HandyReader HD automatically remembers all your settings when powered off e.g. Colour Mode, Zoom Level, LEDs on / off.

## Flip-out stand

To raise the Flip-Out Stand (#15 on Overview Diagram, P5) insert your index finger into the small recess on one side of the stand & lift outwards.

This can be used for reducing the minimum magnification & can aid when handwriting under the unit.

To close the Flip-Out Stand, simply push it back in.

## TV connection

HandyReader HD features the ability to connect to the Composite Video Input socket on your television set.

Simply connect the small end of the supplied TV cable to the Video Output Socket and the Yellow Round Phono (RCA) connector into to the corresponding socket on your TV.

Make sure your television is turned on & select the correct source / input e.g. AV1.

Refer to your television user manual for further details.

NOTE: When connected to a TV screen, there is no output on the HandyReader HD’s screen & you will notice a reduction in frame rate.

## Important precautions

In order to keep your HandyReader HD digital video magnifier in good working order, please read & carefully follow these precautions:

- When not in use, return the magnifier to the supplied storage pouch

- Do not expose the magnifier to high temperatures or direct sunlight

- Keep the magnifier away from moisture, rainwater, liquid and chemicals

- Do not use the magnifier near electronic medical devices

- Only use the magnifier in temperatures between 10°C and 40°C

- Only store the magnifier in temperatures between 5°C and 40°C

- Never attempt to repair or open the magnifier as this will invalidate the warranty and may cause serious damage to the unit

- Only use the supplied mains charger and cables and always replace the internal battery with one of the same specification. Failure to do so may result in injury and/or serious damage to the magnifier

- Before cleaning the magnifier, please disconnect the charger and remove the battery. Only use the supplied lens cloth to clean the magnifier, or a similar soft dry cloth. Never use cleaning materials such as solvents or detergents to clean the magnifier

| Problem | Possible Cause & Solution |
| --- | --- |
| Dark screen with no image | HandyReader HD is turned OFF – Turn ON Battery depleted – Recharge the battery Reading Target is not over text – Relocate over text |
| White screen with no image | Reading Target is not over text – Relocate over text |
| Dim screen | Screen brightness too low. Increase |
| Poor image quality | Dirt on screen – Clean screen with soft dry cloth Dirt on camera – Clean camera with soft dry cloth HandyReader HD is not flat on reading material - Relocate |
| Text too small | Wrong magnification level - Use Zoom In Button |
| Text too large | Wrong magnification level - Use Zoom Out Button. To reduce further, open Flip-Out Stand |
| No image on TV screen | Wrong AV input selected – Refer to TV user manual Poor connection – Carefully check all connections HandyReader HD is turned OFF – Turn ON Battery depleted – Recharge the battery Reading Target is not on text – Relocate over text |

## Technical specification

Screen size: 3.5” HD full colour display

Magnification range: x2 to x24 continuous

Camera pixel size: 1.2 Megapixel

Viewing modes: 9

Lighting: Integrated LEDs with on / off function

Automatically stores last used settings

Composite video output (cable supplied)

Selectable audible feedback

Freeze frame function

Battery life: 3.5 hours depending on settings

Battery type: 1500mAH high-capacity rechargeable Li-ion battery

Dimensions: 115 x 78 x 28mm 4.5 x 3.1 x 1.1 inches

Weight: 130g, 4.6oz (including battery)

## How to contact RNIB

Phone: 0303 123 9999

Email: shop@rnib.org.uk

Address: The Grimaldi Building, 154a Pentonville Road, London N1 9JE.

Online Shop: shop.rnib.org.uk

Email for international customers: kaorders@rnib.org.uk

RNIB Technology Team:

Telephone 0303 123 9999

Email: tfl@rnib.org.uk

## Terms and conditions of sale

This product is guaranteed from manufacturing faults for 24 months from the date of purchase.  If you have any issues with the product and you did not purchase directly from RNIB then please contact your retailer in the first instance.

For all returns and repairs contact RNIB first to get a returns authorisation number to help us deal efficiently with your product return.

You can request full terms and conditions from RNIB or view them online.

© 2024 Royal National Institute of Blind People.  All rights reserved. RNIB Enterprises Limited is a company limited by shares (0887094) registered in England and Wales.  Registered office: The Grimaldi Building, 154a Pentonville Road, London N1 9JE.  RNIB Enterprises Limited is a wholly owned trading subsidiary of the Royal National Institute of Blind People ("RNIB"), a charity registered in England and Wales (226227) and Scotland (SC039316). RNIB Enterprises Limited donates all of its taxable profits to RNIB.

This product is CE marked and fully complies with all applicable EU legislation.

Please do not throw items marked with this symbol in your bin.  Recycle your electricals and electronic devices free at your local recycling centre. Search for your nearest recycling centre by visiting www.recyclenow.com.

### Why recycle?

Unwanted electrical equipment is the UK’s fastest growing type of waste.

Many electrical items can be repaired or recycled, saving natural resources and the environment.  If you do not recycle, electrical equipment will end up in landfill where hazardous substances will leak out and cause soil and water contamination – harming wildlife and human health.

RNIB are proud to support your local authority in providing local recycling facilities for electrical equipment.

To remind you that old electrical equipment can be recycled, it is now marked with the crossed-out wheeled bin symbol. Please do not throw any electrical equipment (including those marked with this symbol) in your bin.

### What is WEEE?

The Waste Electrical or Electronic Equipment (WEEE) Directive requires countries to maximise separate collection and environmentally friendly processing of these items.

### How are we helping?

In the UK, distributors including retailers must provide a system which allows all customers buying new electrical equipment the opportunity to recycle their old items free of charge. As a responsible retailer, we have met the requirements placed on us by financially supporting the national network of WEEE recycling centres established by local authorities. This is achieved through membership of the national Distributor Take-back scheme (DTS).

Date: May 2024.

© RNIB