Mobilux illuminated hand held magnifiers      3.5x/10D, 4x/12D, 5x/16D, 6x/24D

MAG66, MAG67, MAG68, MAG77, MAG93

Please retain these instructions for future reference. These instructions are also available in other formats.

# Importance of a low vision assessment

RNIB and Action for Blind People strongly advise anyone with visual impairment to have a low vision assessment. This is an NHS funded service usually accessed at your local hospital eye clinic or run by a local organisation. Referral to the clinic is usually by your General Practitioner (Doctor), Optometrist or Ophthalmologist. The assessment is free of charge and available to anyone with low vision. You do not have to be registered as sight impaired to access this service.

Low vision means that you will be finding it hard to see things as clearly as you would like to, even after you have had your eyes tested and are wearing the correct spectacles or contact lenses.

At the assessment a qualified low vision practitioner will check your vision and assess your suitability for low vision aids such as magnifiers. These clinics are usually able to provide at least one magnifier to you, free of charge. As magnifiers can be difficult to use initially, the practitioner will help you to get the best out of the equipment they prescribe. If you have been discharged by your eye specialist, you can ask your GP or Optometrist to refer you to the nearest clinic.

# Choosing a magnifier

RNIB's booklet 'Choosing your magnifier' is designed to help people with a visual impairment understand the various types of magnifier that can be used for specific tasks. This booklet is available from RNIB's Helpline in large print, audio or braille, or can be downloaded from rnib.org.uk

# Warning! Important information

Do not leave your magnifier in direct sunlight when it is not being used. The intensifying effect of the lens could become a fire hazard in direct sunlight. Use your protective cover (if provided) and always store in a safe place.

Never look at the sun directly through a magnifying lens as it could damage your eyes.

Do not look directly at the light source in illuminated magnifiers as LEDs are very bright.

Avoid impacting, banging or dropping your magnifier, as well as moisture and high temperatures which may cause damage.

Avoid putting your hands or fingers on the lens of your magnifier - always use the frame or handle.

## Illuminated hand held magnifier

Hand held magnifiers have the benefit of an additional built-in light source, which can be turned on or off. They can be used in either hand, above the writing or object that you want to magnify. Hand held magnifiers can be difficult to use if you have weak or shaky hands or need to keep it in the same position for a period of time. In this instance a stand magnifier may be more suitable.

Most spectacle wearers get best results from a hand held magnifier when using distance, rather than reading spectacles. Place the magnifier flat on the page to be read and then raise the magnifier slowly away from the page until you get the clearest image. Remember that the more powerful the magnifier, the closer you will have to hold it to your eye, and the closer the print will be to the magnifier.

# General description

The Mobilux hand held magnifiers have a lightweight aspheric lens with an innovative Cera-tec® coating that makes them near glass hard. Attachable filters enable you to choose the ideal colour of light to suit your vision requirements from a choice of two colours; yellow or orange. The filters are supplied with the magnifier. The unique step-up power converter produces a brighter and more even illumination over a longer period of time - over 50% brighter than other magnifiers.

Supplied with two AA batteries and a leather protective pouch the Mobilux hand held is available from RNIB in four magnification strengths: 3.5x (rectangle lens) (MAG66), 4x (MAG67), 5x (MAG68) and 6x (round lens) (MAG77). Colour temperature is measured in Kelvin (K), the higher the Kelvin, the brighter the output, and all of these magnifiers use 6,1000K.

## Orientation

### Top

The top of the magnifier, containing the lens, has a white plastic surround. Directly beneath the lens are the manufacturer's details regarding the magnification strength. Below this is the on/off slide-switch for the LED light, which has two tactile ridges across it. The switch and handle are both black, contrasting with the white lens surround. When the on/off slide switch is in the off (lower of two settings) position the word LED can be seen in white on a blue background. When the slide switch is in the on (higher of two settings) position this blue area will disappear under the slide switch.

### Bottom

On the bottom of the magnifier, just underneath the lens is a small LED bulb. This is recessed into a housing that has small notches strategically placed to receive the optional filters included. On the main part of the handle is the battery compartment, and the release catch to remove the cover is found towards the bottom of the handle.

On the base of the handle, is a small hole, which enables you to connect a lanyard (available separately MAG25).

# Getting started

## Inserting/replacing batteries

To insert the batteries, hold the magnifier so the bottom side is facing towards you with the handle nearest to you. Locate the battery cover release catch, which is positioned near the bottom of the handle and gently squeeze it in releasing the catch. You can then remove the battery compartment cover away from the handle.

Insert the two supplied AA batteries into the battery compartment. Insert each battery with the negative (flat) end positioned against the corresponding spring within the compartment. Replace the cover ensuring the locating lugs at the top of the cover line up with the corresponding slots in the battery housing and secure it, you will hear an audible 'click' when the cover is closed correctly.

Replacement batteries are available from RNIB under product code CP03.

Please note: RNIB does not guarantee batteries.

# Using the product

As a general guide, you would use this as a standard hand held magnifier. To check that you are holding it in the correct position, make sure that when the LED light is on that it is shining onto the print that you want to read.

When you are using a hand held magnifier you will need to hold the device in the correct position and distance off the page, so that is in focus. You can do this by placing the magnifier flat on the page and slowly bring it up towards your eye until it becomes in focus. If you can bring the magnifier (and print together) nearer your eye you will be able to see more words in one view. If the image appears upside down, the magnifier and print are too far away from your eye and you will need to bring the magnifier and page together (maintaining the focal position of the device), nearer to your eye. Another way of using your hand held magnifier is to hold the magnifier up to your viewing eye (looking through the centre of the lens), and bring up the reading material until it becomes in focus and is clear to see.

# Hints and Tips

## Aftercare

Proper care of your magnifier means it will provide you with many years of untroubled reading. Clean the lens with a damp cloth (ideally a damp micro-fibre cloth). Never use solvents such as alcohol, benzene or other cleaning agents as these can damage the lens.

You may find small lines that look like hairline cracks at the edge of lightweight lenses or in the plastic frame. These are flow lines that occur during the production process, but they do not affect the function or durability in any way.

Magnifiers usually work best with the spectacles that you have been prescribed by your low vision professional. If you find it difficult to keep your place when reading, try using your finger to mark the start of each line. Holding a magnifier close to your eye and then bringing what you want to see up to it will often help you see more letters and words at a time. Try moving the book or page from side to side rather than moving the magnifier or your eyes. If you find your eyes are getting tired, take a break and start again when you feel better.

## LED lighting

Magnifiers with built-in lighting are nothing new. Some types of magnifiers have used small battery powered ‘traditional’ type light bulbs for many years. Disadvantages with these traditional bulbs were: they could be unreliable: bulbs were difficult to replace; they gave out a yellowish light; and were not energy efficient.

Nowadays, most illuminated magnifiers use Light Emitting Diodes (LEDs) instead of light bulbs. These tiny lights provide a much brighter level of light, tend not to breakdown and do not use much power, meaning that batteries last longer. Modern LEDs can last for over 100,000 hours before they need replacing.

Some makes of magnifiers offer a choice of ‘colour temperature’ which relates to the colour of light emitted. Colour temperatures are measured and described in Kelvin or K. The lower the number (2,700K) will emit a ‘yellow’ light, and the higher the number (6,000K) will emit a bright ‘white’ light.

## Effective lighting

RNIB and Thomas Pocklington's booklet 'Make the most of your sight, improve the lighting in your home' demonstrates good practice by giving ideas, hints and tips. It explains how to light your home more effectively, as well as explaining different types of lighting and their benefits. It is available in large print, audio and braille from RNIB's Helpline or you can download it from our website.

# How to contact RNIB

Phone number: 0303 123 9999

Textphone: 0845 758 5691

Postal address: RNIB, PO Box 173, Peterborough PE2 6WS, U.K.

Email: shop@rnib.org.uk

Web address: www.rnib.org.uk

Email for international customers: exports@rnib.org.uk

## Terms and conditions of sale

This product is guaranteed from manufacturing faults for 24 months from the date of purchase.  If you have any issues with the product and you did not purchase directly from RNIB then please contact your retailer in the first instance.

For all returns and repairs contact RNIB first to get a returns authorisation number to help us deal efficiently with your product return.

You can request full terms and conditions from RNIB or view them online.

Registered Charity No. 226227

Date: March 2014

© RNIB